Sample RFQ from notes:

3/11/25 Chat with Rick Slater

Now shifting gears to ProStamp facility.  Flue baffle.  3” to 4” wide strip of metal.  Sits in exhaust flue.  Slow exhaust down, to keep heat in the tank.  Lots of variants.  But 10 part numbers make up most of the volume.  Brackets gets welded onto the sine wave piece.

1.3 million per year.  Resistance welders currently at Bradford White.  All manual.  They want to automate it. He wants a concept and budgetary quote.

Now insider info.   Currently there is a reg that sunsets gas water heaters sunsets in 2030.   Might not want Cadillac, but somewhat more manual.    Current is 18 people at BFW, want to get down to maybe 4 to 6, and automation.  They do 2 welds per bracket now, with 2 spots.   Maybe better to do projection welding. Ask Jim.

Look at nesting difficulties.  Pack density may be impacted a bunch.  BFW does JIT but with internally built subassemblies. Their hurdle is a bit of space constraints within Hastings plant.  They want to get rid of this welding, but have to find space.

Budgetary.   Get concept.  ProStamp doesn’t have the L bracket stamp die.   So we could get projections on it if needed.

Currently brackets are symmetrical, see if we can make it symmetrically, or possibly be able to do both sides of the same part, ie use 2 and leave 2 projections.   Or we have to do

4/1/25 chat with Shane Ferris, with more info.

Supposedly a 10’year run, because 2035, all gas water heaters are supposed to be done.  Almost a million parts per year.   Tabs are called restrictor plates because they restrict air flow.

3 total restrictor plates in total.  Baffle in the long part. Can interchange baffles and restrictors.  Restrictors always alternate sides.

Look at standardizing on 2 spots per tab.  The tabs appear to be on the same plane, but opposite sides of the wave.

All have same end design with similar lance and crimp.

Ideally, load a wave, hit go and the machine will rotate the wave, auto load the restrictors, and the machine will weld the tabs on.  Operator will then unload.  Beginning and end of the part are on the same plane.  Can locate off the end and the ridge.  The ends and the weld surface are on the same plane.

Clamping will need to be similar to a

Moving headstock and tail stock clamps on a common trunnion rotating headstock.

Welders and feeders will be fixed position, and the part would move left/right to the weld positions.

Look at total volumes to see how to justify it.  Probably 2 operators.  1 running parts and 1 load unload.  Desired TAKT of 25 seconds per part.  Scott Thomas calc is likely 20 sec.

Dual tip welder.

Tabs appear to be same height and length.  So symmetrical, sort of.  Doesn’t matter which side is welded.

In production, so no real SOP or deadline.  Nice to have to reduce labor.

2 baffle lengths and 2 widths.   3 different pitches or wave angles.  So likely 12 total baffles.   But they always have the same left side and right side.

Distance between welds is always 8”.

2 axis pick and place to get tabs from the bowls to the magnetic load area to weld.

Potentially odd numbers of tabs to weld per part.

Sample RFQ-GPT Response:

I have fully reviewed the RFQ document.
I will now begin preparing the Template RFQ Response, as defined in your reference materials.

1. Extract & Summarize Specifications / Requirements

I have organized the information per the Training Spec v1 categories:

| Category | Details from RFQ |
| --- | --- |
| Customer Contact | Not specified. Please confirm contact information. |
| Plant / Install Location | Hastings plant (assumed BFW, Bradford White) – Please confirm. |
| Timing / Schedule | No specific deadline. Project is production support to reduce labor. No hard SOP. |
| PO Date / Kickoff Date / Design Approval | Not specified – Please confirm. |
| Run-Off / FAT Requirements | Not specified – Please clarify acceptance testing requirements. |
| Install / SAT Requirements | Not specified – Please clarify. |
| Parts (Description) | Restrictor plates welded to flue baffle (sine wave strip of metal). |
| Part Numbers / Variations | ~12 baffle variants: 2 lengths, 2 widths, 3 pitches. 3 restrictor plates. Parts interchangeable. |
| Cycle Time / TAKT | Target: 25 seconds per part. Likely achievable ~20 seconds. |
| Annual Quantity | ~1 million parts/year. |
| Operation Steps | 1. Load baffle 2. Auto-rotate and locate 3. Auto-load restrictor plates 4. Weld tabs (dual tip welder) 5. Operator unload |
| Red Rabbit (validation parts)? | Not specified – Please confirm. |
| Robots Needed | 2-axis pick & place for tabs; potentially trunnion rotation of the main part. |
| Inspections | Not specified – Please confirm if weld inspection or verification required. |
| Special Requirements | Symmetrical tab welding; tight nesting considerations; limited footprint; adjustable tooling for all variants. |
| Penalties | Not specified. Please confirm. |
| Spec Name | Not specified. |
| Integration into Existing System | Space constraints noted; no mention of existing line integration. Please clarify. |
| Footprint Constraints | Space constraints within Hastings plant. More specifics needed. |
| CMM Certification | Not specified. Please clarify if any metrology certification required. |
| RCS Coating, Zinc Fasteners, etc. | Not specified. |
| Cost Breakdown Requested | Budgetary quote requested; no formal cost breakdown items specified. |
| 3rd Party Inspections (OSHA/UL?) | Not specified – Please confirm regulatory compliance expectations. |
| Warranty Expectations | Not specified – Please confirm. |

2. Identify Feasibility Risks

Below are the primary risks, gaps, and uncertainties:

- Incomplete Customer/Plant Details: No formal contacts, address, or project ownership identified.

- Ambiguity in Testing and Acceptance Criteria: No definition of FAT, SAT, or part validation.

- Footprint Constraints: Described as “tight,” but no layout provided. This is a critical design constraint.

- Part Variability: 12 baffle variants + 3 restrictor plates + combinations may create significant tooling complexity and changeover time.

- Cycle Time Ambition: 20–25 seconds per part is aggressive considering loading, rotating, locating, auto-feeding, welding, and unloading.

- Projection Welding Feasibility: Conceptually considered but requires validation of part geometry and projections (also unclear if ProStamp has the dies).

- Regulatory Considerations: No mention of OSHA/UL compliance.

- Unclear Scope of Automation: Range from “somewhat manual” to “full automation,” with no clear statement of automation level desired.

3. Suggested Conceptual Approaches

Below are three conceptual approaches derived from Koops experience:

Concept 1: Semi-Automated Trunnion Welding Cell

Description:

- Manual load of baffle onto rotating trunnion fixture.

- Automated pick-and-place of restrictor tabs via 2-axis gantry.

- Fixed dual-tip resistance welder.

- Operator unload.

Pros:
✅ Simpler mechanical design.
✅ Moderate footprint.
✅ Lower cost vs fully automated.
✅ Easier maintenance.

Cons:
❌ Operator required for each cycle.
❌ Changeover tooling may be complex.
❌ TAKT time may be borderline at 25 sec.

Feasibility Risks:

- Part variability requires adaptable fixturing.

- Nesting and space constraints.

Concept 2: Fully Automated Dial Index Cell

Description:

- Dial indexing table with multiple nests for baffle variants.

- Fixed pick-and-place and dual-tip welder.

- Automated unload to outfeed conveyor.

Pros:
✅ Fully automated process (minimal operators).
✅ Consistent throughput.
✅ Good for high volume.

Cons:
❌ Higher complexity & cost.
❌ Changeover between 12 baffle types could be substantial.
❌ Footprint may exceed available space.

Feasibility Risks:

- Requires clear space definition.

- Tooling complexity.

Concept 3: Modular Manual Load Station + Automated Weld Station

Description:

- Manual pre-load of restrictor tabs onto baffle.

- Automated weld cycle only.

- Manual unload.

Pros:
✅ Simplest system.
✅ Low cost.
✅ Smallest footprint.

Cons:
❌ Higher labor.
❌ Potential inconsistent tab placement.
❌ Lower automation benefit.

Feasibility Risks:

- May not achieve desired labor reduction.

4. High-Level Bill of Materials

Separate BOM for Each Concept:

Concept 1: Semi-Automated Trunnion Welding Cell

| Subsystem | Notes |
| --- | --- |
| Trunnion Fixture | Rotating headstock/tailstock clamps |
| 2-Axis Pick & Place | Tab feeder & placer |
| Dual Tip Resistance Welder | Projection weld capability |
| Weld Controller | Integrated controls |
| Operator HMI | Recipe selection, cycle control |
| Safety Enclosure & Light Curtains | Category 3/4 safety |
| Electrical Controls Panel | Allen-Bradley or Siemens controls |
| Tooling & Nests | For 12 baffle variants |
| Pneumatic Systems | Clamps & actuation |

Concept 2: Fully Automated Dial Index Cell

| Subsystem | Notes |
| --- | --- |
| Dial Index Table | Multi-station indexing |
| Multi-Position Fixtures | For baffle variants |
| 2-Axis Pick & Place | Tab feeder |
| Dual Tip Resistance Welder | Projection weld capability |
| Weld Controller | Integrated controls |
| Automated Unload Conveyor | Part transfer out |
| Operator HMI | Recipe management, status monitoring |
| Full Safety Enclosure | Category 3/4 safety |
| Electrical Controls Panel | Advanced controls |
| Pneumatics | Clamping and motion |

Concept 3: Modular Manual Load Station

| Subsystem | Notes |
| --- | --- |
| Weld Fixture | Manual load/unload |
| Dual Tip Resistance Welder | Projection weld |
| Weld Controller | Basic controls |
| Operator HMI | Simple interface |
| Safety Enclosure | Light curtains |
| Electrical Controls Panel | Standard controls |

5. Follow-Up Questions

Critical Questions to Address Before Proceeding:

- Customer Contact and Plant Confirmation:

  - Who is the primary technical and commercial contact?

  - Confirm install location: Hastings plant?

- Space Constraints:

  - Please provide layout drawings or space allocation.

  - Confirm maximum allowable footprint.

- Automation Scope:

  - Confirm desired automation level (semi vs fully automated).

  - Is labor reduction to 2 operators non-negotiable?

- Part Details:

  - Provide CAD files or drawings of baffle and restrictor plates.

  - Confirm final part list and annual volumes.

- Cycle Time Validation:

  - Is 25 sec TAKT a firm requirement?

- Tooling and Changeover:

  - Must the system auto-adjust for all 12 baffle variants?

  - Can some changeover be manual?

- Projection Welding:

  - Do you prefer projection weld over spot weld?

  - Are projection features available in the current design or must we create them?

- Acceptance Testing:

  - What are FAT/SAT criteria?

  - Is there a weld inspection requirement?

- Regulatory & Compliance:

  - Any OSHA/UL/CE certification expectations?

  - Warranty duration required?

- Budget:

  - Are there budgetary constraints or target ranges to guide concept development?

Would you like me to expand any section, prepare budgetary estimates, or refine these conceptual approaches further?