# Example RFQ from Spec document

# Toyota 65PL battery Reserve Bottle

Updated 6/13/25

# General:

- Part is approximately 100mm x 120mm x 265mm (l x w x h) – fairly small.

  - 2 ports (hose) - no inserts

  - Cap to be installed after test

- Molding to be 1+1 family mold.

- Cycle time:  Molding @ 45 seconds

- Include data logging (to USB) of significant parameters (weld, insertion, and test)

- Option #1:  Entire new cell (capital and tooling)

- Option #2:  Tooling only to fit in existing cell (TOM1009)

# Part:

# Automation:

## Concept:

## Mold:

- Robot #1 (Fanuc) to service the mold.

  - EOAT “double sided” to tend mold and assembly stations.

- Robot #1 picks bottles from mold.

- Rejected parts from molding process (RJG) or assembly to be placed in Reject Bin

- Breakdown:

  - Tooling:  Design, EOAT

  - Capital:  robot, pedestal, controls

## Welder:

- Robot #1 to pick & place bottles in welder.

  - Bottles will need to be “flipped” into weld position.

- New rotary table hot plate welder – similar to existing

  - Need to review size of part – similar to P552 (S2266)

  - Includes servo controls (top, bottom, and hot plate), pneumatic and electrical for nests, vacuum for upper nest.

    - 7kN capacity for top & bottom servos

  - 8 zone heat control for hot plate

  - Force Transducer for upper platen

- Weld nests

  - Profile plate in sections to allow adjustments to fit part without additional machining.

  - Slides under protrusions

  - Part ejection cylinder in upper

  - Part temperature sensor (IR) in bottom nests

- Hot plate

  - Weld plates - aluminum with Teflon coating

- Rejected part placed in Reject Bin

- Breakdown:

  - Capital:  Welder (base, controls, programming)

  - Tooling:  Design, Nests, Melt Plates

  - Option:  40 spare weld plates

## Cooling:

- Robot #1 to pick & place bottles from cooling nests.

- 4 Nests with forced air, blowing chilled air (Move-N-Cool) into fillneck

- Breakdown:

  - Tooling:  Design, Part Nests

  - Capital:  Move-N-Cool, Base, Controls, etc.

## Test / datecode

- Robot #1 to pick & place bottles from cooling

- Include a part temperature sensor (Raytek).

- Leak Test

  - Fillneck has a vent port, so custom fitting is required.

  - FasTest or CTS seals for 2 outlet ports.

  - Test #1, Test pressure relief port.  Supply air at 15 psi.  Pressure sensor to detect back pressure.

  - Test #2, leak test at TBD (assume 100 kPa), TBD sccm using LeakMaster

  - Test #1 and Test #2 can be performed simultaneously due to cycle time.

- Breakdown:

  - Tooling:  Robot EOAT, Design, tooling plate, nesting, clamps, seals

  - Capital:  Robot, LeakMaster, Reject bin, Laser marker and balance of equipment.

## Cap installation (at Insertion station):

- Robot #1 to present bottle to torque equipment

- Bowl feeder to feed caps to deadnest

- Robot #2 (LR Mate) to move cap to bottle

- Caps rotated/torqued in place

- Breakdown:

  - Tooling:  Design, tooling plate, nesting, bowl feeder, inline, cap grippers/mandrel

  - Capital:  Base, controls, cap torquer

## WIP:

- Drawer/nest to be able to put assembled bottles back into system for test only.

- Breakdown:

  - Tooling:  Part nest

  - Capital:  Drawer, Base

## Packaging:

- Robot #1 to place rejected bottles in Reject Bin

- Robot #1 to hold good bottles for datecode with laser marker, place bottles in good totes.

- Positions for 2 totes (1 being filled, 1 full/empty)

  - Totes to be changed by operator.

- Breakdown:  all capital

## Bins

- Scrap bin with chute for robot

  - Locked access from outside of cell

- WIP bin with chute for robot

  - Access from outside of cell

- Breakdown:  all capital

Sample RFQ response:

User: for the concepts, please disregard the retrofitting information and just focus on the full cell applications

ChatGPT said:

I have reviewed the RFQ document in detail. Below is a structured draft RFQ response per the template, focusing only on the full cell (Option #1) approach as you requested.

🟢 RFQ Response: Toyota 65PL Battery Reserve Bottle

1. Extract & Summarize Specifications / Requirements

Below, I have organized the provided requirements aligned to our Training Spec v1 checklist:

| Category | Details |
| --- | --- |
| Customer Contact | Not specified. ⚠️ Must confirm. |
| Plant / Install Location | Not specified. ⚠️ Must confirm. |
| Timing / PO Date / Kickoff | Not specified. ⚠️ Must confirm project timeline and milestones. |
| Design Approval / FAT Requirements | Not specified. ⚠️ Must clarify expectations for design review, runoff, FAT, SAT. |
| Part | Reserve Bottle approx. 100mm x 120mm x 265mm; 2 ports (hose); no inserts; cap installed after test. |
| Part Variations / Part Numbers | Not specified. ⚠️ Must confirm if variations exist or will be introduced. |
| Cycle Time | Molding cycle time specified at 45 seconds. |
| Annual Quantity / Demand | Not specified. ⚠️ Must confirm. |
| Description of Requirements | Entire new cell with molding tending, welding, cooling, leak testing, cap insertion, date coding, and packaging. |
| Operation Steps | 1. Mold tending & part pick 2. Hot plate welding 3. Cooling with chilled air 4. Leak test (2 steps) 5. Cap installation 6. Date coding 7. Packaging |
| Robots Needed | - Robot #1: Fanuc (large robot) performing nearly all operations - Robot #2: LR Mate for cap handling |
| Inspections / Testing | - Weld data logging - Leak testing (2-stage) - Date coding |
| Special Requirements | - Data logging to USB - IR temperature sensing - Custom seals and nests - Reject bin handling |
| Footprint Constraints | Not specified. ⚠️ Must confirm available floor space and ceiling height. |
| Integration into Existing System | This approach is new cell only. |
| CMM Certification / RCS / Fasteners | Not specified. ⚠️ *Must confirm any standards for documentation, coatings, fasteners. |
| Penalties / Warranty | Not specified. ⚠️ Must confirm. |
| Cost Breakdown Requested | Yes, costs separated by Tooling and Capital per sub-process. |
| 3rd Party Inspections | Not specified. ⚠️ *Must confirm (UL, OSHA, etc.). |
| Warranty Length Expected | Not specified. ⚠️ Must confirm. |

2. Feasibility Risks

Below are potential red flags and ambiguities that must be clarified before moving to engineering:

- Plant Location / Environment: No information on installation site (space, utilities, ambient conditions).

- Cycle Time Alignment: While molding time is 45 seconds, the cumulative downstream process time (welding, cooling, leak test, capping, marking) appears similar or longer. The RFQ does not provide clear TAKT/cycle expectations per station.

- Part Handling Precision: Flipping bottles for welding and precise nesting in multiple stations will require careful EOAT design.

- Leak Test Specs Incomplete: Leak test pressures and leak rates ("TBD")—must be specified.

- Integration Requirements: No mention of MES or plant-level communications (barcode scanning, production tracking).

- Data Logging / IT: Format, frequency, and IT security policies for data logging to USB unspecified.

- Safety / Compliance: No explicit requirements for safety guarding standards, UL, CE marking.

- No confirmation of Annual Volume: Critical to determine throughput expectations and robot sizing.

- No footprint constraints provided: May impact cell design/layout.

3. Suggested Conceptual Approaches (Full Cell)

Note: All concepts assume new automation cell only.

Concept 1: Dual Robot Cell with Central Rotary Welding and Inline Process Flow

✅ Description

- Robot #1 (Fanuc) performs mold tending, welding loading/unloading, cooling, leak test loading/unloading, date coding, and good/reject handling.

- Robot #2 (LR Mate) performs cap placement and torquing.

- Inline process with rotary table hot plate welder.

- Fixed cooling nests.

- Standalone leak test station.

- Laser date marking integrated inline.

Pros:

- Fewer robots (2 total) to reduce complexity.

- Streamlined linear flow from molding to packaging.

- Reuses proven Koops approach to rotary welding (similar to P552).

Cons:

- Robot #1 highly utilized – risk of cycle time bottleneck.

- Complex EOAT with flipping and multiple nests.

- Potential for longer cycle times per part.

Feasibility Concerns:

- Confirmed leak test spec and cycle time needed to validate throughput.

Concept 2: Three-Robot Cell with Parallel Processing

✅ Description

- Robot #1 (Fanuc): Mold tending and welding only.

- Robot #2 (Fanuc LR Mate): Cooling and leak testing.

- Robot #3 (LR Mate): Cap insertion and date coding.

- Parallel stations for welding and leak test to decouple time.

- Cooling nests in-line between welding and leak test.

Pros:

- Cycle time risk mitigated by decoupling leak test and welding.

- Dedicated robot for delicate cap insertion improves reliability.

- Simplifies EOAT design (specialization).

Cons:

- Higher capital cost (additional robot).

- Larger footprint needed.

Feasibility Concerns:

- Footprint constraints unknown.

- Leak test spec TBD.

4. High-Level Bill of Materials

Below is a preliminary BOM for each concept.
(Note: quantities and suppliers are subject to detail engineering.)

Concept 1 BOM

| Subsystem | Description |
| --- | --- |
| Robot #1 | Fanuc 6-axis robot + controller |
| Robot #2 | Fanuc LR Mate + controller |
| EOAT | Dual-sided gripper, flipping mechanism |
| Hot Plate Welder | Rotary table welder w/ servo platens |
| Cooling Station | Forced air nests, Move-N-Cool system |
| Leak Test Station | LeakMaster w/ custom fixtures |
| Cap Insertion | Bowl feeder, torque drive |
| Laser Marker | Inline date coder |
| Controls | Central PLC, HMI, safety devices |
| Guarding | Perimeter fencing, interlocks |
| Data Logging | Industrial PC w/ USB export |
| Packaging | Totes, reject bins |

Concept 2 BOM

| Subsystem | Description |
| --- | --- |
| Robot #1 | Fanuc 6-axis robot + controller |
| Robot #2 | Fanuc LR Mate + controller |
| Robot #3 | Fanuc LR Mate + controller |
| EOAT | Specialized grippers per robot |
| Hot Plate Welder | Rotary table welder w/ servo platens |
| Cooling Station | Forced air nests, Move-N-Cool system |
| Leak Test Station | LeakMaster w/ custom fixtures |
| Cap Insertion | Bowl feeder, torque drive |
| Laser Marker | Inline date coder |
| Controls | Central PLC, HMI, safety devices |
| Guarding | Perimeter fencing, interlocks |
| Data Logging | Industrial PC w/ USB export |
| Packaging | Totes, reject bins |

5. Follow-Up Questions

The following must be answered before proceeding:

- Customer Contact and Install Site

  - What is the plant location and environment (floor space, utilities, ceiling height)?

  - Who is the primary project contact?

- Timing

  - Confirm project milestones: PO date, kickoff, design approval, FAT, SAT.

- Cycle Time & Volume

  - What is the target TAKT time per finished part?

  - What is the annual production volume?

- Leak Test Specification

  - Confirm leak test pressure (Test #2): currently TBD kPa.

  - Confirm leak rate threshold (sccm): currently TBD.

- Data Logging

  - What file format, frequency, and retention policy for USB data logging?

- Integration Requirements

  - Will this system need MES connectivity, barcode tracking, or ERP interfaces?

- Footprint

  - Are there any constraints on footprint or height?

- Compliance

  - Any specific 3rd-party inspections (UL, OSHA)?

  - Required documentation packages (CMM certification, manuals)?

- Standards

  - Any preferences for coatings (RCS), fastener types, warranty expectations?

Please review this draft carefully.

When ready, I can help you:

- Revise the concepts,

- Develop budgetary pricing,

- Refine the BOM detail,

- Draft customer-facing presentations.

Would you like to proceed with refining this or clarifying outstanding questions?